|  | Outdoor Access Point | OST | Extreme | I-tek | Door sanchar | Fore | GNDEC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | The Access Point should have minimum 1 Port 10/100/1000Mb POE in Ethernet port. |  |  | The Access Point should have minimum 1 Port 10/100/1000Mbps in Ethernet port. |  |  | This is just to explain that we need access point with POE input |
| 2 | 802.11n Access Point should be able to power up using standards 802.3af/at POE input, and at the same time operate in full MIMO mode. |  |  |  |  |  |  |
| 3 | AP should have Dual Radios to support 2.4 GHz & 5Ghz concurrent users with 802.11 a/b/g/n capability. AP Must support 3x3 or 2x3 or 2x2 MIMO with 2 Radio Chain |  |  |  |  |  |  |
| 4 | AP should be able to handle upto 200 Concurrent users. |  |  |  |  |  |  |
| 5 | AP should provide minimum transmission power of 25 dBm in 2.4Ghz and 5 Ghz. |  |  | AP should provide minimum transmission power of 20 dBm in 2.4Ghz and 5 Ghz. |  |  | This will be low as per our requirement. Specially in hostels |
| 6 | Wireless Interface: Dual radio; 802.11a/b/g/n/ac; 2.4Ghz and 5 Ghz. |  |  | Remove ac “Wireless Interface: Dual radio; 802.11a/b/g/n/ac; 2.4Ghz and 5 Ghz” ---- |  |  | AC is now a days very common |
| 7 | SSID support : 16 BSSID (8 BSSID per Radio) |  |  |  |  |  |  |
| 8 | AP should support 6.5Mbps – 130Mbps (20MHz), upto 300Mbps (40MHz) datarates in 802.11n. |  |  |  |  |  |  |
| 9 | The access point should support 802.1q VLAN tagging |  |  |  |  |  |  |
| 10 | Antenna: Integrated/External for Sectorial/Omni- directional (as specified in BOQ) coverage, with min 4 dB aggregate Gain for 2.4Ghz and 5Ghz both. |  |  | Antenna: Integrated/External for Sectorial/Omni- directional (as specified in BOQ) coverage, with min 4 dB aggregate Gain for 2.4Ghz and 5Ghz. |  |  | Unable to understand the query |
| 11 | Should support the operating temp -10° to 55° C and and humidity: 15 to 95% non- condensing | Should support the operating temp 0° to 50° C and and humidity: 15 to 95% non- condensing |  |  | Change humidity: 15 to 90% non- condensing |  | 50 Degree temperature is very normal in summers now a days |
| 12 | AP Must be IP67 certified for outdoor deployment. AP must be outdoor rated and no AP will be accepted which is indoor and installed in outdoor casing. |  |  |  |  |  |  |
| 13 | The access point should support following security mechanism: WEP, WPA-PSK, WPA-TKIP, WPA2 AES, 802.11i. |  |  |  |  |  |  |
| 14 | System should support Authentication via 802.1X, Local (controller based) authentication database, support for RADIUS and Active Directory. |  |  |  |  |  |  |
| 15 | Web User Interface (HTTP/S) • CLI (Telnet/SSH), SNMP v1,2,3 |  |  |  |  |  |  |
| 16 | Should be managed by Controller or standalone if Required |  |  |  |  |  |  |
| 17 | WEEE/RoHS compliance •Wi-Fi Alliance certified |  |  |  |  |  |  |
| 18 | Should be WPC approved; ETA certificate to be Enclosed |  |  | Should be WPC approved/ETA certificate to be Enclosed |  |  | It is mandatory as per govt. norms being an educational institute |